### SIMATS ENGINEERING

#### ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

QUESTIONS: 5	Total Marks:50

Annexure A

## Scenario-Based

Code of Conduct:

I JAYA HARINI.B.R[192565031] (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

JAYA HARINI.B.R[192565031]

Annexure A

## Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine the network address and broadcast address for each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26 Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine the network address and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

#### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |

# ANSWERS:

## Scenario 1

Given Network: 192.168.100.0/24

| Depar tment | Sub net Ma sk | Networ k Addres s | First Host | Last Host | Broadc ast Addres s | Usa ble Ho sts |
| --- | --- | --- | --- | --- | --- | --- |
| School of Compu ting | /25 | 192.168. 100.0 | 192.168. 100.1 | 192.168. 100.126 | 192.168. 100.127 | 126 |
| School of Electro nics | /26 | 192.168. 100.128 | 192.168. 100.129 | 192.168. 100.190 | 192.168. 100.191 | 62 |
| School of Mecha nical Engine ering | /27 | 192.168. 100.192 | 192.168. 100.193 | 192.168. 100.222 | 192.168. 100.222 | 30 |

Remaining Unused IP Address Range

      - 192.168.100.224 – 192.168.100.255

      - Remaining block size = 32 IP addresses (equivalent to one

/27 subnet)

## Scenario 2

Given Network: 10.10.0.0/16

| Departm ent | Sub net Mas k | Networ k Addres s | First Host | Last Host | Broadc ast Addres s | Usa ble Host s |
| --- | --- | --- | --- | --- | --- | --- |
| Software Develop ment | /24 | 10.10.0. 0 | 10.10.0. 1 | 10.10.0. 254 | 10.10.0. 255 | 254 |
| Quality Assuranc e | /25 | 10.10.1. 0 | 10.10.1. 1 | 10.10.1. 126 | 10.10.1. 127 | 126 |
| Human Resource s | /26 | 10.10.1. 128 | 10.10.1. 129 | 10.10.1. 190 | 10.10.1. 191 | 62 |
| Executiv e Manage ment | /27 | 10.10.1. 192 | 10.10.1. 193 | 10.10.1. 222 | 10.10.1. 223 | 30 |

Remaining Unused IP Address Range

      - 10.10.1.224 – 10.10.255.255

## Scenario 3

Given IPv6 Address: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

- Compressed Notation 2001:db8:abcd::/64

- Expanded Notation 2001:0db8:abcd:0000:0000:0000:0000:0000/64

- Network Prefix (/64)

2001:0db8:abcd:0000::/64

Network Prefix = 2001:0db8:abcd:0000

- Total Host Addresses Supported A /64 subnet leaves 64 host bits. Total addresses:

2⁶⁴ = 18,446,744,073,709,551,616 addresses

- Total Possible Host Addresses 18,446,744,073,709,551,616 (2⁶⁴) addresses

## Scenario 4 Routing Table

Destination Network Prefix

192.168.1.0	/24

192.168.2.0	/24

192.168.3.0	/24

Destination IP: 192.168.2.45

## Answers

- Destination Subnet 192.168.2.0/24

- Matching Routing Table Entry 192.168.2.0/24

- Next-Hop Network/Interface

Forward the packet through the interface connected to the 192.168.2.0/24 network.

- Default Route

If no routing table entry matches, the router forwards the packet using the default route:

0.0.0.0/0

Scenario 5

Administration LAN Network: 192.168.10.0/26

  - Network Address: 192.168.10.0

  - Broadcast Address: 192.168.10.63

  - Valid Host Range:

o 192.168.10.1 – 192.168.10.62

  - Default Gateway:

o 192.168.10.1

Suggested IP Addresses

  - PC1 → 192.168.10.2

  - PC2 → 192.168.10.3

  - PC3 → 192.168.10.4

Finance LAN

Network: 192.168.10.64/26

  - Network Address: 192.168.10.64

  - Broadcast Address: 192.168.10.127

  - Valid Host Range:

o 192.168.10.65 – 192.168.10.126

  - Default Gateway:

o 192.168.10.65

Suggested IP Addresses

  - PC1 → 192.168.10.66

  - PC2 → 192.168.10.67

  - PC3 → 192.168.10.68