# SIMATS ENGINEERING

### ASSESSMENT TOOL 3

#### COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0714

#### COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30% Software:Cisco Packet Tracer,Wireshark

##### QUESTIONS: 5	Total Marks:50

Annexure A

## Debugging & Optimisation Task

###### Code of Conduct:

I Jaya Harini BR (192565031) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

###### Signature of the student:

###### Jaya Harini B R

Annexure A

### Debugging & Optimisation Task

- An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

  - Identifying the network address, broadcast address, and host range for each subnet.

  - Calculating the number of usable and unused IP addresses.

  - Identifying inefficient subnet allocations.

  - Debugging the addressing plan by locating subnet allocation mistakes.

  - Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

#### ANSWER:

##### Initial Configuration

| Device | IPv6 Address | Prefix Length |
| --- | --- | --- |
| PC0 | 2001:db8::ff00:42:8329 | /64 |
| PC1 | 2001:db8:0:0:1::1 | /64 |

##### Expanded IPv6 Addresses Device A

Compressed Address:2001:db8::ff00:42:8329

Expanded Address:2001:0db8:0000:0000:0000:ff00:0042:8329

##### Device B

Compressed Address:2001:db8:0:0:1::1

Expanded Address: 2001:0db8:0000:0000:0001:0000:0000:0001

##### Network Prefix Calculation

Since the prefix length is /64, the first four hextets represent the network portion.

##### Device A

2001:db8:0:0::/64

##### Device B

2001:db8:0:0::/64

Initially, both devices belong to the same subnet.

##### Debugging Scenario

To demonstrate IPv6 troubleshooting, the IPv6 address of PC1 was intentionally modified. Modified IPv6 Address of PC1

2001:db9:0:0:1::1/64

Now,

##### PC0 Network

2001:db8:0:0::/64

##### PC1 Network

2001:db9:0:0::/64

Since the network prefixes are different, both devices belong to different subnets, causing communication failure.

##### Optimized IPv6 Configuration

After debugging, the IPv6 configuration was corrected. Device	Correct IPv6 Address	Prefix PC0	2001:db8::ff00:42:8329	/64

PC1	2001:db8:0:0::20	/64

Both devices now belong to the same subnet.

#### AFTER DEBUGGING:

##### Reply from 2001:DB8::20

Packets: Sent = 4 Received = 4

Lost = 0

Communication was successfully restored after assigning the correct IPv6 address.

Available Host Addresses Prefix Length:/64

Remaining Host Bits:128 − 64 = 64 Available Host Addresses:2^64

= 18,446,744,073,709,551,616

##### Approximately 18.4 Quintillion host addresses are available in a /64 IPv6 subnet.

#### RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justificatio n | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |